OCP POC

LVMS & LOCAL STORAGE

IMPLEMENTATION GUIDE

Adding 1TB Local Disks + LVM Storage Operator for AIDA Workloads

| Cluster | ocp-poc.amer.epiqcorp.com |
| --- | --- |
| OCP version | 4.21.12 |
| Storage type | LVMS (LVM Storage Operator) — local NVMe/SSD disks |
| Disk per worker | 1TB raw unformatted disk (/dev/sdb) |
| Workers covered | worker-1 through worker-5 (5 nodes) |
| StorageClass | lvms-vg1 (RWO — ReadWriteOnce) |
| Access mode | ReadWriteOnce — best for databases and high I/O |
| Existing NFS | nfs-storage (RWX) — remains for shared workloads |
| Requested by | Greg — AIDA data platform (Solr, Qdrant, PostgreSQL) |
| Document date | 3 July 2026 |

# 1. Overview

Greg has requested local high-performance block storage for AIDA workloads. The existing NFS storage (nfs-storage StorageClass) provides ReadWriteMany (RWX) shared access which is suitable for Solr and Qdrant. However for PostgreSQL and other database workloads that require high I/O performance, local disks via LVMS provide significantly better throughput.

LVMS (LVM Storage Operator) is a Red Hat operator that manages local disks on worker nodes using LVM thin provisioning. It creates a StorageClass that allows pods to request persistent volumes from the local disks directly — no NFS overhead, no network latency.

## 1.1  Storage comparison

|  | NFS (existing) | LVMS (new) | Winner |
| --- | --- | --- | --- |
| Performance | Network-bound — ~1Gbps | Local disk — full disk speed | LVMS |
| Access mode | RWX — multiple pods | RWO — one pod per volume | NFS for shared |
| Best for | Solr, Qdrant, shared data | PostgreSQL, databases, high I/O | Depends on workload |
| Snapshots | No | Yes — via VolumeSnapshot | LVMS |
| Thin provisioning | No | Yes — overprovisioning 10x | LVMS |
| Node failure | Data survives | Data on that node is unavailable | NFS |
| Setup complexity | Already done | This guide | N/A |

## 1.2  Recommended storage mapping for AIDA

| AIDA component | StorageClass | Reason | Size |
| --- | --- | --- | --- |
| PostgreSQL | lvms-vg1 | High I/O database — benefits most from local disk | 100Gi |
| Solr | nfs-storage | Shared index access — needs RWX | 1Ti |
| Qdrant | nfs-storage or lvms-vg1 | Vector DB — local disk preferred if RWO is acceptable | 200Gi |
| General/misc | nfs-storage | Shared workloads — keep on NFS | As needed |

# 2. Prerequisites

## 2.1  What is needed from the Compute Pod team

Before starting this guide the following must be in place:

| Requirement | Details |
| --- | --- |
| 1TB raw disk on each worker VM | Add as a second disk (/dev/sdb) in VMware — thin provisioned is fine. Do NOT format or partition it. LVMS must see it as completely raw. |
| 5 workers total | worker-1 through worker-5. worker-5 (management node) already added. |
| OpenShift cluster running | OCP 4.21.12 — all operators healthy. |
| Red Hat subscription active | Required to pull LVMS operator images from registry.redhat.io. |
| Cluster admin kubeconfig | Required to install the operator and create resources. |

CRITICAL: The 1TB disk MUST be completely raw — no partition table, no filesystem, no LVM signature. If the disk has any existing data LVMS will not claim it. Power off each worker (cordon and drain first), add the disk in VMware, power back on.

## 2.2  Adding the raw disk to each worker — VMware procedure

Repeat this for each of the 5 workers. Do one at a time — never drain more than one worker simultaneously.

export KUBECONFIG=~/ocp-poc-install/auth/kubeconfig # Step 1 — cordon the worker oc adm cordon worker-1.ocp-poc.amer.epiqcorp.com # Step 2 — drain the worker oc adm drain worker-1.ocp-poc.amer.epiqcorp.com \ --ignore-daemonsets \ --delete-emptydir-data # Wait for drain to complete — you will see: node/worker-1 drained # Step 3 — in VMware: # Power off worker-1 # Edit VM settings → Add New Device → Hard Disk # Size: 1000 GB, Thin Provision # Do NOT format or partition — leave completely raw # Power on worker-1 # Step 4 — uncordon oc adm uncordon worker-1.ocp-poc.amer.epiqcorp.com # Step 5 — verify disk is visible and raw oc debug node/worker-1.ocp-poc.amer.epiqcorp.com -- chroot /host lsblk # Should show sdb with no partitions and no filesystem # Repeat for worker-2, worker-3, worker-4, worker-5

NOTE: After uncordoning each worker verify it returns to Ready status before moving to the next: oc get nodes ¦ grep worker

# 3. Install the LVMS Operator

The LVM Storage Operator (LVMS) is available in the Red Hat OperatorHub. It can be installed via the web console or CLI. The CLI method is documented here.

## 3.1  Create the openshift-storage namespace

export KUBECONFIG=~/ocp-poc-install/auth/kubeconfig # Create namespace cat <<EOF ¦ oc apply -f - apiVersion: v1 kind: Namespace metadata: name: openshift-storage labels: openshift.io/cluster-monitoring: 'true' EOF # Create OperatorGroup cat <<EOF ¦ oc apply -f - apiVersion: operators.coreos.com/v1 kind: OperatorGroup metadata: name: openshift-storage-operatorgroup namespace: openshift-storage spec: targetNamespaces: - openshift-storage EOF

## 3.2  Subscribe to the LVMS operator

cat <<EOF ¦ oc apply -f - apiVersion: operators.coreos.com/v1alpha1 kind: Subscription metadata: name: lvms-operator namespace: openshift-storage spec: channel: stable-4.21 name: lvms-operator source: redhat-operators sourceNamespace: openshift-marketplace EOF

## 3.3  Wait for operator to install

# Watch operator pod come up watch -n 10 'oc get pods -n openshift-storage' # Should see something like: # lvms-operator-xxxx 1/1 Running 0 2m # Also check ClusterServiceVersion oc get csv -n openshift-storage # Should show: lvms-operator Succeeded

NOTE: If the operator stays in Pending or the CSV shows Failed, check that the Red Hat subscription pull secret is valid. An expired trial subscription will prevent image pulls from registry.redhat.io.

# 4. Create the LVMCluster

The LVMCluster resource tells LVMS which disks to use on which nodes, how to configure LVM, and what StorageClass to create. This is the most important configuration step.

## 4.1  Create the LVMCluster resource

cat <<EOF ¦ oc apply -f - apiVersion: lvm.topolvm.io/v1alpha1 kind: LVMCluster metadata: name: ocp-lvmcluster namespace: openshift-storage spec: storage: deviceClasses: - name: vg1 storageClassName: lvms-vg1 thinPoolConfig: name: thin-pool-1 sizePercent: 90 overprovisionRatio: 10 deviceSelector: paths: - /dev/sdb EOF

## 4.2  Configuration explained

| Field | Value | Meaning |
| --- | --- | --- |
| storageClassName | lvms-vg1 | The StorageClass name Greg uses in his PVC specs |
| thinPoolConfig.sizePercent | 90 | 90% of each disk goes to the thin pool — 10% reserved for LVM metadata |
| thinPoolConfig.overprovisionRatio | 10 | Can provision up to 10x the physical space — thin provisioning allows overcommit |
| deviceSelector.paths | /dev/sdb | The raw disk added to each worker — must match exactly |

## 4.3  Verify LVMCluster is ready

# Check LVMCluster status oc get lvmcluster -n openshift-storage # Should show: Ready # Check all nodes have the topolvm agent running oc get pods -n openshift-storage # Should see topolvm-node-* pods on each worker # Check StorageClass was created oc get storageclass # Should show lvms-vg1 # Detailed cluster status oc describe lvmcluster ocp-lvmcluster -n openshift-storage

WARNING: If any worker does not have /dev/sdb visible the LVMCluster will show degraded for that node. Verify all disks were added correctly before creating the LVMCluster.

# 5. Test the Storage

Before giving Greg the StorageClass, verify it works by creating a test PVC and pod.

## 5.1  Create a test PVC

cat <<EOF ¦ oc apply -f - apiVersion: v1 kind: PersistentVolumeClaim metadata: name: lvms-test-pvc namespace: eaida spec: accessModes: - ReadWriteOnce storageClassName: lvms-vg1 resources: requests: storage: 10Gi EOF # Check it bound oc get pvc lvms-test-pvc -n eaida # STATUS should be: Bound

## 5.2  Create a test pod to verify write access

cat <<EOF ¦ oc apply -f - apiVersion: v1 kind: Pod metadata: name: lvms-test-pod namespace: eaida spec: containers: - name: test image: registry.access.redhat.com/ubi9/ubi-minimal:latest command: ['/bin/sh', '-c', 'echo LVMS test > /data/test.txt && cat /data/test.txt && sleep 3600'] volumeMounts: - name: data mountPath: /data volumes: - name: data persistentVolumeClaim: claimName: lvms-test-pvc EOF # Wait for pod to run oc get pod lvms-test-pod -n eaida # Check the output oc logs lvms-test-pod -n eaida # Should print: LVMS test

## 5.3  Clean up test resources

oc delete pod lvms-test-pod -n eaida oc delete pvc lvms-test-pvc -n eaida

# 6. Create PVCs for AIDA

Based on Greg's request, create the following PVCs in the eaida namespace. PostgreSQL moves to LVMS for better I/O performance. Solr and Qdrant can stay on NFS or optionally move to LVMS.

## 6.1  PostgreSQL — move to LVMS (high priority)

The existing postgres-data-disk-pv on NFS should be replaced with a LVMS-backed PVC for better database performance. Note: this requires Greg to migrate the existing postgres data.

# New postgres PVC on LVMS cat <<EOF ¦ oc apply -f - apiVersion: v1 kind: PersistentVolumeClaim metadata: name: postgres-data-lvms-pv namespace: eaida spec: accessModes: - ReadWriteOnce storageClassName: lvms-vg1 resources: requests: storage: 100Gi EOF oc get pvc postgres-data-lvms-pv -n eaida

## 6.2  Optional — Qdrant on LVMS

If Greg confirms Qdrant only runs on a single pod (not shared across pods), it can move to LVMS for better vector search performance.

cat <<EOF ¦ oc apply -f - apiVersion: v1 kind: PersistentVolumeClaim metadata: name: qdrant-data-lvms-pv namespace: eaida spec: accessModes: - ReadWriteOnce storageClassName: lvms-vg1 resources: requests: storage: 200Gi EOF

## 6.3  Current PVC inventory after LVMS addition

| PVC name | Size | StorageClass | Notes |
| --- | --- | --- | --- |
| solr-data-disk-pv | 1Ti | nfs-storage | Stays on NFS — Solr needs RWX shared access |
| qdrant-data-disk-pv | 200Gi | nfs-storage | Can optionally move to LVMS if RWO acceptable |
| postgres-data-disk-pv | 100Gi | nfs-storage | Legacy — replace with postgres-data-lvms-pv |
| postgres-data-lvms-pv | 100Gi | lvms-vg1 | New — high performance local disk for Postgres |
| qdrant-data-lvms-pv | 200Gi | lvms-vg1 | Optional — higher performance for vector search |

# 7. How Greg Uses LVMS in AIDA Deployments

Greg references the StorageClass in his deployment specs. Here are examples for the main AIDA components.

## 7.1  PostgreSQL deployment example

# In Greg's postgres StatefulSet or Deployment: volumeClaimTemplates: - metadata: name: postgres-data spec: accessModes: ["ReadWriteOnce"] storageClassName: lvms-vg1 # <-- use this for postgres resources: requests: storage: 100Gi

## 7.2  Solr deployment example (stays on NFS)

# In Greg's Solr StatefulSet: volumeClaimTemplates: - metadata: name: solr-data spec: accessModes: ["ReadWriteMany"] # RWX — must use NFS storageClassName: nfs-storage # <-- use this for Solr resources: requests: storage: 1Ti

## 7.3  Node affinity with LVMS

LVMS volumes are local to the node they are created on. If a pod using an LVMS PVC is scheduled to a different node it will fail to mount. Kubernetes handles this automatically via topology constraints, but Greg should be aware:

- Once an LVMS PVC is bound to a node, the pod using it will always schedule on that same node

- This is by design — local storage is faster because it is tied to one node

- If that worker node goes down, the pod cannot start until the node recovers

- NFS volumes do not have this restriction — they can move between nodes freely

NOTE: For stateful workloads like PostgreSQL that should stay on one node anyway, this is not a problem. Greg's node labeling strategy (datalake, backend, frontend) works well with this — each component stays on its designated node.

# 8. Day-2 Operations

## 8.1  Checking LVMS health

# Check LVMCluster status oc get lvmcluster -n openshift-storage # Check all topolvm pods are running oc get pods -n openshift-storage # Check disk usage per node oc get lvmvolumegroup -n openshift-storage # Check all LVMS PVCs oc get pvc -A ¦ grep lvms

## 8.2  Expanding an LVMS volume

LVMS supports online volume expansion — no downtime required.

# Edit the PVC to request more storage oc edit pvc postgres-data-lvms-pv -n eaida # Change: storage: 100Gi to storage: 200Gi # Or patch it directly oc patch pvc postgres-data-lvms-pv -n eaida \ --type merge \ -p '{"spec":{"resources":{"requests":{"storage":"200Gi"}}}}' # Verify expansion oc get pvc postgres-data-lvms-pv -n eaida # CAPACITY column should update to 200Gi

## 8.3  Adding a new LVMS disk to a worker

If a worker runs out of disk space, add a second raw disk in VMware (/dev/sdc) and update the LVMCluster to include it:

# Update LVMCluster to add the new disk oc edit lvmcluster ocp-lvmcluster -n openshift-storage # Add /dev/sdc to deviceSelector.paths: # deviceSelector: # paths: # - /dev/sdb # - /dev/sdc <-- add this # LVMS will automatically extend the volume group # No downtime required

## 8.4  Checking disk space

# Check physical disk usage on each node oc debug node/worker-1.ocp-poc.amer.epiqcorp.com -- chroot /host df -h # Check LVM volume group space oc debug node/worker-1.ocp-poc.amer.epiqcorp.com -- chroot /host vgdisplay # Check thin pool usage oc debug node/worker-1.ocp-poc.amer.epiqcorp.com -- chroot /host lvdisplay

## 8.5  Removing an LVMS PVC

Because the StorageClass uses Retain policy, deleting a PVC does not delete the underlying data:

# Delete PVC — data is retained on disk oc delete pvc postgres-data-lvms-pv -n eaida # The PV will show as Released oc get pv ¦ grep lvms # To permanently delete the data, delete the PV too oc delete pv <pv-name>

# 9. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| LVMCluster shows degraded | One or more workers missing /dev/sdb or disk has existing data | Check lsblk on affected node — wipe disk with wipefs -a /dev/sdb if needed |
| PVC stays in Pending | LVMS pods not running or LVMCluster not Ready | oc get pods -n openshift-storage and oc describe lvmcluster |
| Pod fails to mount volume | Pod scheduled on wrong node — LVMS volumes are node-local | Check pod events: oc describe pod <name> — ensure node affinity is set |
| topolvm-node pod CrashLooping | Disk not found or permission issue | oc logs on the crashing pod — check /dev/sdb exists on that node |
| Operator install fails | Expired Red Hat subscription — can't pull images | Renew subscription and update pull secret in the cluster |
| CSV shows Failed | OperatorGroup or namespace issue | Check oc get operatorgroup -n openshift-storage |

Document notes

This guide covers LVMS installation on OCP 4.21.12 with 5 worker nodes at 10.76.175.24-28. LVMS operator version matches the stable-4.21 channel. The overprovisionRatio of 10 means up to 10TB of PVCs can be provisioned from 1TB of physical disk using thin provisioning — actual data written must not exceed physical capacity. Monitor thin pool usage regularly using the commands in Section 8.